[image: image3.jpg]



מפרט מס': 92/13
1. כללי
1) משטרת ישראל מעוניינת לרכוש אנטנות עבור התקנה בתורני קשר, 
רכבים,ממסרי קשר  ומכלולי קשר ע"פ דרישות טכניות המפורטות במפרט זה.
2) כל הפריטים המופיעים במפרט יהיו אך ורק מוצרי מדף.

2. סוגי אנטנות

1) אנטנה לאתר
2) אנטנה  - VHF כב"א/מד"א
3) אנטנה לממסר
4) אנטנה כיוונית
5) אנטנה db5
6) אנטנה רב-תדר "פנטום"
7) ערכת חווט לאנטנה "פנטום"
8) אנטנה  כנפון                     

3. דרישות טכניות

1) אנטנה לאתר    
1. ELECTRICAL SPECIFICATION
2. Antenna Type                                                 Omni
3. Frequency Band                                 746-869MHz
4. Beamwidth ,Horizontal ,degrees                       360

5. Gain , dbi                                                         12.0
6. Beamwidth ,Vertical ,degrees                            6.0

7. Beam Tilt , degrees                                                0

8. VSWR |  Return Loss , db                       1.5:1| 14.0

9. Input Power , maximum , watts                         500

10. Polarization                                                 Vertical

11. Impedance , ohms                                                50

12. Lightning Protection                               dc Ground

2. MECHANICAL SPECIFICATION
1. Color                                                    Horizon blue      

2. Connector Interface                      7-16 DIN Female
3. Connector Location                                      Bottom

4. Connector Quantity                                                1

5. Working temperature                   From -35˚ to +60˚

3. DIMENSIONS
1. Length                                   4495.8 mm  |  177.0 in

2. Outer Diameter                      76.2 mm      |     3.0 in

3. Net Weight                            17.2 kg        |    38.0 lb

4. RADIATION PATTERNS
	Vertical Pattern
	Horizontal Pattern


	[image: image1.png]w0010 o

Freq: 820 MHz, Tilt: 0




	[image: image2.png]w000 10

i
W0 gt

Freq: 820 MHz, Tilt: 0






[image: image4.png]H - PLANE




התמונה להמחשה בלבד
2) אנטנה VHF כב"א/מד"א    

1. ELECTRICAL SPECIFICATION
1. Frequency range                               144–174 MHz
2. Input impedance                                          50 ohm

3. V.S.W.R                                                       < 1:1.5

4. Beamwidth – 3 dBi                                             74˚
5. RF power                                         100  - 150 watt 
6. Polarization                          Vertical or Horizontal

7. Gain                                                               1 dBd

8. Type of connection                                   N female

9. All metal parts                                    Dc grounded

2. MECHANICAL SPECIFICATION
1. Mounting hole        From 33 to 60 mm master tube

2. Boom material            Aluminun alloy DIN 3.3215
3. Elements  material      Aluminun alloy DIN 3.2306

4. Wind resistance                                        160 km/h

5. Working temperature                  From -35˚ to +60˚

6. Mounting u-bolt                                Stainless steel

7. All screws and nuts                           Stainless steel

3. DIMENSIONS
1. Width x Length                        
  615 x 820 mm

2. Net weight                                
        abt. 2400g

4. RADIATION PATTERNS
[image: image5.png]E - PLANE




[image: image6.png]1.5

VSWR
1.3

1.2
1.1

VSWR CURVE

144

150

166

162 MHz

174





[image: image7.png]



התמונה להמחשה בלבד

3) אנטנה לממסר 
1. ELECTRICAL SPECIFICATION
1. Antenna Type                                                 Omni 

2. Frequency range                               740–870 MHz
3. Input impedance                                          50 ohm

4. V.S.W.R                                                       ≤ 1.5:1

5. Beamwidth – 3 dB                                             15˚

6. Maximum power                                       250 watt

7. Polarization                                                Vertical 

8. Gain -minimum                                              8 dBi
9. Type of connection                                   N female
10. All metal parts                                    Dc grounded

2. MECHANICAL SPECIFICATION

1. Mounting master tube    Form 40 to 65 mm diameter
2. Radome material                                        Fiberglass
3. Maximum rated wind velocity                    216 km/h

4. Dimensions                                      Ø 52 x 1800 mm
5. Net weight                                
          abt. 3000g

6. Working temperature                     From -35˚ to +60˚
7. Mounting u-bolt                                   Stainless steel

8. All screws and nuts                              Stainless steel[image: image8.jpg]








[image: image9.jpg]



                                    התמונה להמחשה בלבד

4) אנטנה כיוונית
1. ELECTRICAL SPECIFICATION
1. Antenna Type                                                  Yagi 

2. Frequency range                               740–870 MHz

3. Beamwidth                                               130 MHz

4. Horizontal Beamwidth                                       48˚        

5. Vertical Beamwidth                                           40˚   

6. F/B ratio                                                           ≥ 15
7. Input impedance                                          50 ohm

8. V.S.W.R                                                          ≤ 1.5

9. Maximum power                                       100 watt

10. Polarization                          Vertical or Horizontal
11. Gain-minimum                                          11.2 dBi   
12. Type of connection           N female or Customized
13. All metal parts                                    Dc grounded

2. MECHANICAL SPECIFICATION 

1. Length                                                            80 cm
2. Weight                                                         0.38 Kg
3. Mounting Mast Diameter                        Ø 40 to 50 
4.  Working temperature                     From -35˚ to +60˚

5. Mounting u-bolt                                   Stainless steel

6. All screws and nuts                              Stainless steel

    



[image: image10.png]



                                       התמונה להמחשה בלבד
5) אנטנה db5
1. ELECTRICAL SPECIFICATION
1. Antenna Type                                                 Omni

2. Frequency range                               740–870 MHz

3. Gain                                                            5-6 dBi   
4. Vertical Beamwidth                                           35˚   

5. V.S.W.R                                                          ≤ 1.5

6. Impedance                                                   50 ohm

7. Polarization                                                Vertical 

8. Maximum power                                       100 watt

9. Connector                                                 N female

10. All metal parts                                    Dc grounded

2. MECHANICAL SPECIFICATION 

1. Dimensions                                        Ø 33 x720mm
2. Weight                                                         0.78 Kg
3. Pole Diameter                                   Ø 40 to 50 mm
4. Working temperature                     From -35˚ to +60˚

5. Mounting u-bolt                                   Stainless steel

All screws and nuts                              Stainless steel                          .6
[image: image11.jpg]



התמונה להמחשה בלבד

6) אנטנה רב-תדר "פנטום"
1. ELECTRICAL SPECIFICATION
1. Antenna Type                      Multiband                           

2. Frequency range         698–960 MHz , 1710-2700 MHz
3. V.S.W.R                                                     

1. 698-806 MHz                                       < 2.5:1.0

2. 806-960 MHz                                       < 2.0:1.0

3. 1710-2110 MHz                                   < 2.0:1.0

4. 2110-2170 MHz                                   < 2.5:1.0

5. 2300-2700 MHz                                   < 2.0:1.0

4. Nominal Peak Gain

1. 698-960 MHz  -minimum                           3 dBi                     

2. 1710-1990 MHz-minimum                         5 dBi                     

3. 2100-2700 MHz -minimum                        4 dBi                     
5. Average Efficiency

1. 698-960 MHz                           90% (max : 96%)                     

2. 1710-1990 MHz                       91% (max : 95%)            

3. 2100-2700 MHz                       80% (max : 86%)                
6. Input impedance                                                   50 ohm

7. Maximum power                                                100 watt

8. Polarization                                                         Vertical 

9. Pattern                                                    Omnidirectional

10. Terminations                    NMO or P-Mount w/Type n(f)
2. MECHANICAL SPECIFICATION
1. Color                                                         Black or White  

2. Radome material                                      High Heat ABS

3. Dimensions                       3.3" (8.38cm) x 1.44"(3.65cm)

4. Working temperature                           From -40˚ to +85˚

3. RADIATION PATTERNS
[image: image12.jpg]Aaimuth Elevation

750 MHz 849 MHz

Elevation saimth Elevation
960 MHz 1710 bz

evation Azimuth

1950 MHz 2700 Mz




[image: image13.jpg]



התמונה להמחשה בלבד

התמונה להמחשה בלבד

7) ערכת חווט לאנטנה "פנטום"
            ערכת החווט מכילה את הפריטים הבאים :

1) מחבר מיני UHF.
2) מחבר מתאם לאנטנה עם כבל LMR195 באורך 5 מטר.
3) גומייה לאום
4) גומייה לגג
5) אום למחבר אנטנה
6) משחת סילקון
[image: image14.jpg]


[image: image15.jpg]



[image: image16.jpg]Typical VSWR*

*VSWR measured with 5m of cable

e e W @ w m w w W W



[image: image17.emf]






                                                התמונה להמחשה בלבד
8) אנטנה  כנפון
1. ELECTRICAL SPECIFICATION

1. Antenna Type                                                 Omni

2. Frequency range                              700–960 MHz

3. Gain                                                                1 dBi

4. Vertical Beamwidth                                           35˚   

5. V.S.W.R                                                      ≤ 2.5:1

6. Impedance                                                   50 ohm

7. Polarization                                                Vertical 

8. Maximum power                                         30 watt

9. Connector                                                 N female

2. MECHANICAL SPECIFICATION 

1. Height                                                         130mm

2. Width                                                            17mm

3. Thickness                                                     2.5mm

4.  Operating  Temp.                                -40˚ to +85˚

5. Material                                                            TPE

6. Colour                                                            Black



.ג               CABLE  DATA  
1. Type                                                                SR35 
2. Diameter                                                       3.5mm

3. Length                                                                 5m

                                 4.   Termination                                             MPL plug         






















     .ד RADIATION PATTERNS    






                            התמונה להמחשה בלבד     
4. אחריות

1) הזוכה מתחייב לתת שירותי תחזוקה ללא תמורה לפרק זמן של 12 חודשים החל ממועד אספקת האנטנות למרכז שירות.
2) האחריות תכלול : 

1) שמירה על רמת הביצועים הטכניים של האנטנה.
2) שמירה על איכות החומרים המכנים של האנטנה (פלסטיקה ומתכת).

3) הזוכה במכרז ישא על חשבונו בכל ההוצאות הכרוכות בביצוע תיקוני התקלות באנטנות ,כולל אספקת חלפים מקורים בלבד ללא פגיעה במבנה הפיסי של האנטנה.
4) האחריות לא תכלול תקלות הנוגעות משימוש בניגוד להנחיות מפורשות שניתנו על ידי הזוכה במכרז או במקרה של נזק בזדון. 
5) הזוכה במכרז יתקן כל תקלה תוך שבעה ימים ממועד מסירת ההודעה על ידי המזמין.

במידה שהאנטנה לא תתוקן בפרק זמן זה ,תסופק למשטרת ישראל אנטנה חלופית תקינה.
5. אספקה

1) הזוכה יספק את האנטנות למשטרת ישראל – מחסן מחלקת הקשר מטה ארצי רמלה.

2) באחריות הזוכה קבלת /החזרת האנטנות ממחסן הקשר במטה ארצי רמלה לצורך תיקונם
6. אריזה

1) אנטנה לאתר – באריזת קרטון.

2) אנטנה VHF כב"א/מד"א – באריזת קרטון.
3) אנטנה לממסר – באריזת קרטון.
4) אנטנה כיוונית – באריזת קרטון.
5) אנטנה 5db – באריזת קרטון.
6) אנטנה רב תדר "פנטום" – באריזת ניילון.
7) ערכת חווט לאנטנת "פנטום" – באריזת ניילון.

8) אנטנת כנפון – באריזת ניילון. 
7. אבטחת איכות 

1) בדיקות לאישור דגם
1) המציעים יגישו תוך 7 ימים מדרישה דוגמה עבור כל אחד מסוגי האנטנות לצורך בדיקות.
אין צורך להגיש דוגמה עבור פריט מס' 1.

2) האנטנות יבדקו לעמידה בדרישות הטכניות הנדרשות בסעיף ג' בהתאם לדף נתוני יצרן (PDF) שיצרף המציע להצעתו.
2) ביקורת קבלה

       כל משלוח יבחן לגבי התאמה לדגם שאושר , שלמות האריזה וסוג 
       האנטנה.

  פנחס      אהרון 

מח' הקשר ואלקטרוניקה 

.ג





מחבר מיני UHF





משחת סילקון





אום למחבר אנטנה





מחבר מתאם לאנטנה עם כבל LMR195 באורך 5 מטר





גומייה לאום





גומייה לגג








PAGE  
1

