SECTION 5.2 – LIGHT VEHICLE RADIOGRAPHY SYSTEM

CONTENTS

SECTION 5.2 – LIGHT VEHICLE RADIOGRAPHY SYSTEM

The Light Vehicle Radiography System

1.	General

2.	System’s infrastructures

The Light Vehicle Radiography System – Technology and equipment supplied by others

- General - for contractor information only

  - The purpose of the site is the Radiography systems designated for cargo and vehicle inspections and creating images to be interpreted by the operators.

  - Stationary - The facility of the Light Vehicle X-Ray system shall be a stationary one, in a designated fixed structure including the radiation enclosure with radiation shielding walls and doors, operational and managerial rooms.

  - Conveyor - The system shall be based on a Conveyor architecture based on a vertical radiography layout.

  - High Energy X-Ray - The Light Vehicle Radiography system shall generate one or more images using high energy X-Ray radiation.

  - Material Discrimination – The proposed imaging systems shall include material discrimination features enabling differentiation between organic and inorganic materials.

  - System’s main components:

The system shall include, at least:

    - An X-Ray emission subsystem:

    - An X-Ray detection subsystem, sensitive to X-Rays, which produces digitized information corresponding to the detected radiation;

    - Command and control subsystem; including the processing subsystem to process the digitized information and output this information in the form of high-definition images;

    - The Light Vehicle Radiography system shall operate in a way of providing automatic co-ordination of these subsystems, independently of any other facility systems, including the power supply system.

  - System’s minimal performances are defined in this document.

  - System description - A detailed description will be provided in Bidder's proposal, including specific reference to the following issues:

    - System’s architecture, subsystems

    - System’s operation

    - Image Workstation: hardware, software, operation

    - Data Architecture, linkage to all subsystems

    - Material Discrimination feature

    - Image Processing Tools

    - Systems Performance

- System’s Infrastructures

  - The System technology will be supplied, installed, start up and commissioned by others.

  - The system will be installed in the site, located as shown in the general layout and according to system supplier and contractor coordination.

  - The System’s infrastructures should be prepared by the contractor, including integration with the following:

    - Civil works – all disciplines

    - Electrical supply

    - All Traffic issues including controls

    - Communication

    - PA system

    - Fire detection and extinguishing

    - Integration with FCS system

    - Water and sanitary